- void osalAddTasks( void )  //OSAL_serialApp.c

{

#if defined( MT_TASK )

osalTaskAdd( MT_TaskInit, MT_ProcessEvent, OSAL_TASK_PRIORITY_LOW );

#endif

osalTaskAdd( OnBoard_TaskInit, OnBoard_ProcessEvent, OSAL_TASK_PRIORITY_LOW );

osalTaskAdd( nwk_init, nwk_event_loop, OSAL_TASK_PRIORITY_MED );

osalTaskAdd( APS_Init, APS_event_loop, OSAL_TASK_PRIORITY_LOW );

osalTaskAdd( ZDApp_Init, ZDApp_event_loop, OSAL_TASK_PRIORITY_LOW );

osalTaskAdd( SerialApp_Init, SerialApp_ProcessEvent, OSAL_TASK_PRIORITY_LOW );

}

**********************************************************************************

- void OnBoard_TaskInit( byte task_id )  //OnBoard.c

{

OnBoard_TaskID = task_id;      /* task_id is a global variable, here it indicate OnBoard_ProcessEvent */

} /* OnBoard_TaskInit() */

/*********************************************************************

- void SerialApp_Init( byte task_id )   //SerialApp.c

{

……

RegisterForKeys( task_id );

}

**********************************************************************************

- byte RegisterForKeys( byte task_id )

{

// Allow only the first task

if ( registeredKeysTaskID == NO_TASK_ID )

{

registeredKeysTaskID = task_id;    /* 把registeredKeysTaskID 設為SerialApp_ProcessEvent的Task_ID*/

}

}

**********************************************************************************

- UINT16 OnBoard_ProcessEvent( byte task_id, UINT16 events )  //OnBoard.c

{

if ( events & KEYPRESS_DEBOUNCE_EVT )

{

read_keys();  // Get key info

KBIntSetup( true );  // Re-enable interrupts

// Return unprocessed events

return (events ^ KEYPRESS_DEBOUNCE_EVT);    /*回到osal_start_system 無窮迴圈中*/

}

} /* OnBoard_ProcessEvent() */

**********************************************************************************

- INTERRUPT_KEYBD()  /* 中斷處理常式 */    //OnBoard.c

{

…..

…..

{

// Wait a little while to avoid key bounce

if ( OnBoard_TaskID != NO_TASK_ID )

osal_start_timerEx( OnBoard_TaskID, KEYPRESS_DEBOUNCE_EVT, 20 );

/* 20 ms 後產生  KEYPRESS_DEBOUNCE_EVT，將OnBoard_TaskID放進 ready list */

}

}

**********************************************************************************

- byte osal_start_timerEx( byte taskID, UINT16 event_id, UINT16 timeout_value )  //OSAL_Timers.c

{

osalTimerRec_t *newTimer;

………..

newTimer = osalAddTimer( taskID, event_id, timeout_value );

}

**********************************************************************************

- UINT16 SerialApp_ProcessEvent( byte task_id, UINT16 events )  //SerialApp.c

{

switch ( *msgPtr )

{

case KEY_CHANGE:

SerialApp_HandleKeys( msgPtr[KEY_CHANGE_SHIFT_IDX], msgPtr[KEY_CHANGE_KEYS_IDX] );

break;

**********************************************************************************

- void SerialApp_HandleKeys( byte shift, byte keys )   //SerialApp.c

if ( keys & EVAL_SW1 )

{

char theMessageData[] = "Hello World";

MT_SerialSendZAppMsg((byte*)&theMessageData, (byte)osal_strlen(theMessageData));

}

**********************************************************************************

- byte OnBoard_SendKeys( byte shift, byte keys )

{

……..

osal_msg_send( registeredKeysTaskID, msgPtr, 3 );   /*丟MSG給serialApp*/

}

**********************************************************************************

/*

* Routine to poll for key presses

*/

- static void read_keys( void )

{

….

if ( OnBoard_SendKeys( ((keys & EVAL_SW6) ? true : false), keys ) != ZSuccess )

{

}

}

**********************************************************************************

- void osal_start_system( void )

{

for(;;)

{

activity = false;

activeTask = osalNextActiveTask();

if ( activeTask )

{

StoreDisableInts;

events = activeTask->events;

// Clear the Events for this task

activeTask->events = 0;

RestoreInts;

if ( events != 0 )    /*等到serialapp為activetask的時候*/

{

// Call the task to process the event(s)

if ( activeTask->pfnEventProcessor )

{

retEvents = (activeTask->pfnEventProcessor)( activeTask->taskID, events );

// Add back unprocessed events to the current task

StoreDisableInts;

activeTask->events |= retEvents;

RestoreInts;

activity = true;

}

}

}

}

}

}

1

2

3

4

6

中斷

7

12

5

11

10

20ms later&

輪到檢查OnBoard的event

8

9

Initial

輪到檢查SerialApp的event